Q-Learning Maze Navigation

Ackermann-Style Robot in ROS 2 and Gazebo

Revanth Sai Srinivas Akula

# 1. Summary

A small robot learns to drive through a 5 × 5 maze using tabular Q-learning. Learning is done in a fast simulation and saved as a Q-table. The robot then uses the table in Gazebo (ROS 2 Humble). The robot never reads the wall map while driving. It uses only its position, the goal it is given, live LIDAR and its own memory.

- Both mazes are solved on every start/goal pair by a shortest path (2400 / 2400).

- Training takes about 25 seconds per maze.

- A table is only trusted for the maze it was trained on. A maze signature checks this.

# 2. Mazes and Results

The grid is 5 × 5 and each cell is 1.0 m × 1.0 m. Directions: 0 = East, 1 = North, 2 = West, 3 = South. Ties are broken in that order. Each maze has its own world file and its own Q-table.

|  | maze.world | maze2.world |
| --- | --- | --- |
| Inner walls | 12 | 14 |
| Maze signature | bb421745fc82 | 338ec0596cb2 |
| Dead-end cells | (0,0) (2,2) (2,4) (3,4) (4,4) | (0,4) (1,2) (1,4) (2,1) (3,3) (4,0) (4,1) |
| Longest shortest path (any two cells) | 10 moves | 15 moves |
| Shortest path (0,0) to (4,4) | 8 moves | 8 moves |
| Q-table states | 2456 | 2452 |
| Passes to finish training | 15 | 15 |
| Training time | 21.9 s | 24.5 s |
| Start/goal/heading cases reached | 2400 / 2400 | 2400 / 2400 |
| Cases on a shortest path | 2400 / 2400 (100%) | 2400 / 2400 (100%) |

Inner walls of maze.world (each sits between two cells): (0,0)-(0,1), (1,0)-(1,1), (1,2)-(1,3), (2,1)-(2,2), (2,2)-(2,3), (2,2)-(3,2), (2,3)-(2,4), (2,4)-(3,4), (3,0)-(3,1), (3,1)-(4,1), (3,2)-(4,2), (3,4)-(4,4).

Inner walls of maze2.world: (0,2)-(1,2), (0,4)-(1,4), (1,1)-(1,2), (1,1)-(2,1), (1,2)-(2,2), (1,4)-(2,4), (2,0)-(2,1), (2,1)-(3,1), (2,3)-(3,3), (3,1)-(3,2), (3,2)-(3,3), (3,3)-(3,4), (4,0)-(4,1), (4,1)-(4,2).

Note: 2400 cases = 600 start/goal pairs × 4 starting headings. Both mazes are fully connected, so every case can be solved.

# 3. Robot

| Item | Value |
| --- | --- |
| Wheelbase / track width | 0.213 m / 0.222 m |
| Wheel radius | 0.0505 m |
| Max steering angle | 30° (min turning radius 0.369 m) |
| Speed range | −0.25 to 0.60 m/s |
| LIDAR | 360 beams, 0.15 m to 6.0 m, height 0.1505 m |
| Control rate | 10 Hz |
| Drive mode in demo | diff (turns on the spot) |

# 4. Q-Learning

After every move one Q-value is improved:

new Q = old Q + learning_rate × ( reward + discount × best next Q − old Q )

| Setting | Value |
| --- | --- |
| Learning rate | 0.1 |
| Discount factor | 0.95 |
| Epsilon (exploration) | 1.0 falling to 0.10 over 20 passes |
| Maximum passes / steps per episode | 400 / 100 |
| Stop rule | 2 perfect checks in a row (checked every 5 passes) |
| Reward: reach goal | +100 |
| Reward: normal step | −1 |
| Reward: hit a wall | −10 |
| Reward: timeout | −20 |
| Distance bonus each step | +0.5 closer to goal, −0.5 farther |
| Revisit penalty (−5) | Off during training |

- Exploring starts: every start/goal/heading and every first move is tried in each pass, so no state keeps a wrong value.

- Safe saving: the trained policy is compared with the true shortest path (BFS). If any case is not shortest, the table is not saved.

# 5. Unique Key Generation

State key. Every situation is written as five whole numbers. This tuple is the key of the Q-table (a dictionary). The value is four Q-values [East, North, West, South].

(robot_x, robot_y, goal_x, goal_y, heading)

- Before learning, the trainer loops over 25 goals × 24 robot cells × 4 headings = 2400 keys, each starting at [0, 0, 0, 0].

- A few extra "robot on goal" keys are added while learning (the final table has 2456 and 2452 states).

- The same situation always gives the same key, and different situations never share one. At run time the robot builds the key from its own position, the given goal and its heading, then looks it up.

Example: (0, 0, 4, 4, 0) is robot at (0,0), goal (4,4), facing East. Its row is [51.71, 0, 0, 0]. North is 0 because there is a wall above (0,0).

Maze signature. A second key says which maze a table belongs to. The inner walls are written as text (e.g. "0,0-0,1"), sorted, joined, hashed with SHA-256, and the first 12 characters are kept. Moving one wall changes the signature completely. The signature is saved inside the table, and the robot warns at start-up if it does not match the world file.

# 6. DFS Backup

DFS (depth-first search) is in maze_memory.py and is called from run_policy.py. It walks into unvisited cells and backtracks from dead ends. It needs no map.

- Normal case: the Q-table drives. DFS is not used.

- DFS takes over when: the state is not in the table, the table's move points into a wall LIDAR sees, or the robot enters one cell more than 3 times.

- After that, DFS finishes the run and a warning is printed.

Results on maze.world: correct table gives 600/600 with 0 DFS steps. With one wall moved and the old table, DFS still finishes 600/600.

# 7. Controller Settings

| Setting | Value |
| --- | --- |
| Driving / turning speed | 0.15 m/s / 0.6 rad/s |
| Position / angle tolerance | 0.08 m / 0.03 rad |
| Wall threshold | 0.80 m (wall about 0.45 m, open about 1.45 m) |
| Goal radius | 0.20 m |

# 8. How to Run

python3 -m robot_controller.train

ros2 launch robot_description gazebo.launch.py x_cell:=0 y_cell:=0 heading:=0 drive_mode:=diff

python3 -m robot_controller.run_policy --start 0 0 --goal 4 4 --heading 0

For maze2, set RL_MAZE_WORLD to maze2.world and RL_Q_TABLE to its own table file before training and running.